CP5 → CP6 → CP6.5
Navigation Codebase — Complete Understanding Guide

NeuroGait Quadruped Navigation  |  Implementation branch
Go2 robot · Isaac Lab · skrl PPO · CNN+MLP policy

────────────────────────────────────────────────────────────────────────────────

Section 1 — The Big Picture

The entire stack is a hierarchical control system: a high-level navigation policy (what direction should I go?) drives a frozen locomotion policy (how do I move my legs?). The two policies never see each other's internals — they communicate only through a 3D velocity command: [vx, vy, heading].

TRAINING LOOP  (scripts/train.py)

│

gym.make("NeuroGait-Navigation-CP65-v0")

│

┌────────────┼────────────┐

▼            ▼            ▼

SceneCfg    ObservationsCfg  RewardsCfg

(obstacles,  (what policy    (what behavior

cameras,     sees)           is rewarded)

robot)           │                │

│            ▼                ▼

│     NavigationPolicy   9 reward terms

│     (CNN+MLP, 1615→3)  (multiplicative core)

│            │

│            ▼

│     PreTrainedPolicyAction

│     (frozen 12-joint loco, 235→12)

│            │

└────────────┼────────────┘

▼

Isaac Sim / PhysX

(50 Hz physics,

5 Hz nav steps via decimation=40)

IMPORTANT: The navigation policy runs at 5 Hz (every 40 physics steps = 0.2 s). The locomotion policy runs at 50 Hz internally. Isaac Lab's 'decimation' parameter controls this: the manager-based env calls env.step() every 40 physics steps.

The Two-Level Hierarchy

Level 1 — Navigation Policy (TRAINED HERE, 5 Hz)

Input:  1615-dim obs  [40×40 grid | 9 waypoints | 3 velocity | 3 gravity]

Output: 3-dim action  [vx ∈ [-1,1] m/s | vy ∈ [-1,1] m/s | heading ∈ [-π,π] rad]

Model:  CNN branch (grid → 64-dim) + MLP branch (scalars → 32-dim) → 3

Level 2 — Locomotion Policy (FROZEN, 50 Hz, pre-trained Go2 checkpoint)

Input:  235-dim obs   [joint positions, velocities, body velocity, commands, ...]

Output: 12-dim        [target joint angles for 12 Go2 joints]

Loaded: PreTrainedPolicyActionCfg in navigation_env_cfg.py (path to .pt file)

The navigation policy's [vx, vy, heading] output is injected into the locomotion policy's 'velocity_commands' observation slot by PreTrainedPolicyAction. The locomotion policy THINKS a joystick is giving it commands — it has no idea a navigation policy is in the loop.

Section 2 — Every File Explained

config/go2/ — Environment Configuration

These files define WHAT the training environment looks like. They are pure Python dataclasses decorated with @configclass — no computation happens here. Isaac Lab reads them at gym.make() time and builds the actual env.

navigation_env_cfg.py

The main config file. Contains the full inheritance chain from the base env down to CP65. Every @configclass here is one registered task.

ManagerBasedRLEnvCfg                        (Isaac Lab)

└── NavigationBaseEnvCfg                  (navigation_base_env_cfg.py)

└── NeuroGaitNavigationGo2BaseEnvCfg

├── NeuroGaitNavigationCP1EnvCfg     (CP3/CP4 rule-based)

└── NeuroGaitNavigationCP5EnvCfg     (CP5 — trained policy)

├── NeuroGaitNavigationCP5EnvCfg_PLAY

└── NeuroGaitNavigationCP6EnvCfg     (CP6 — A* replan)

├── NeuroGaitNavigationCP6EnvCfg_PLAY

└── NeuroGaitNavigationCP65EnvCfg  (CP6.5 — generator)

└── NeuroGaitNavigationCP65EnvCfg_PLAY

Key things set in __post_init__:

• RayCasterCamera (depth sensor) pointed downward at 30° pitch

• PreTrainedPolicyActionCfg — loads frozen locomotion checkpoint

• ObservationsCfg — wires occupancy_grid_obs_cp5 + future_waypoints_obs + velocity + gravity

• RewardsCfg — either CP5RewardsCfg or CP6RewardsCfg

• EventsCfg — reset event that either randomizes obstacles (CP6) or generates scene (CP6.5)

cp5_rewards.py

Dataclass holding reward term configs for CP5. Separated from the env config to keep navigation_env_cfg.py clean. Contains 7 terms (additive).

| Term | Function | Weight |
| --- | --- | --- |
| velocity_toward_goal | cp5_reward_velocity_toward_goal | +10.0 |
| goal_proximity | cp5_reward_goal_proximity | +0.1 |
| goal_reached | cp5_reward_goal_reached | +20.0 |
| collision | cp5_penalty_collision_velocity_scaled | -5.0 |
| stuck | cp5_penalty_stuck | -0.3 |
| heading | cp5_reward_heading | +0.1 |
| smoothness | cp5_penalty_smoothness | -0.01 |

cp6_rewards.py

CP6 reward terms (9 terms, multiplicative core). Replaces additive CP5 velocity reward with a product of forward/lateral/heading Gaussians — prevents crab-walking.

| Term | Function | Weight |
| --- | --- | --- |
| navigation_core | cp6_reward_navigation_core | +10.0 |
| path_following | cp6_reward_path_following | +0.5 |
| goal_proximity | cp5_reward_goal_proximity | +0.1 |
| goal_reached | cp5_reward_goal_reached | +50.0 |
| slow_near_goal | cp6_reward_slow_near_goal | +3.0 |
| graduated_clearance | cp6_penalty_graduated_clearance | -0.05 |
| collision | cp5_penalty_collision_velocity_scaled | -1.5 |
| stuck | cp6_penalty_stuck_v2 | -0.3 |
| smoothness | cp6_penalty_smoothness_2nd_order | -1.0 |

__init__.py  (config/go2/)

Registers all tasks with gymnasium. The key line is gym.register(id=..., env_cfg_entry_point=..., skrl_cfg_entry_point=...). Task IDs registered:

NeuroGait-Navigation-Unitree-Go2-Play-v1    (CP1 play only)

NeuroGait-Navigation-CP5-v0                 (CP5 train)

NeuroGait-Navigation-CP5-Play-v0            (CP5 play)

NeuroGait-Navigation-CP6-v0                 (CP6 train)

NeuroGait-Navigation-CP6-Play-v0            (CP6 play)

NeuroGait-Navigation-CP65-v0                (CP6.5 train)  ← train this

NeuroGait-Navigation-CP65-Play-v0           (CP6.5 play)

mdp/ — Observations, Rewards, Events

These files contain the actual COMPUTATION. Functions here are referenced by the config dataclasses in config/go2/ via ObsTerm, RewTerm, EventTerm, and DoneTerm wrappers. Isaac Lab calls them every step.

mdp/observations.py

All observation functions. The most important ones for CP5/CP6/CP6.5:

occupancy_grid_obs_cp5(env) → Tensor (E, 1600)

Converts depth camera output to a 40×40 binary occupancy grid.

Robot is at cell (row=10, col=20) — asymmetric: 2m behind, 6m ahead.

Depth values outside [0.05, 2.0]m are masked.

Cells are 0.20m × 0.20m → total coverage: 8m × 8m.

future_waypoints_obs(env) → Tensor (E, 9)

Returns the NEXT 3 waypoints relative to robot body frame.

Each waypoint = (dx_body, dy_body, dist) → 3 scalars × 3 waypoints = 9.

Advances waypoint index when robot is within 0.3m of current waypoint.

robot_velocity_obs(env) → Tensor (E, 3)

Returns [vx, vy, yaw_rate] in body frame from robot state.

_cp5_init_waypoint_state(env) → None

Lazy init: called by EVERY reward and obs function that needs waypoints.

On first call: runs A* (CP5/CP6) or detects generator has already run (CP6.5).

Stores env._cp5_waypoints (E, W, 2) world coords as env attribute.

If env._cp5_waypoints already exists (generator set it) — returns immediately.

This is the KEY integration point between generator and rewards.

_cp5_reset_waypoint_state(env, env_ids) → None

Called by EventTerm on episode reset to clear wp_idx, prev_dist, action history.

mdp/rewards.py

All CP5 and CP6 reward functions. Shared helper: _cp5_current_wp_and_dist() reads env._cp5_waypoints and returns current waypoint + distance.

CP5 reward functions:

cp5_reward_velocity_toward_goal  → cos(Δθ) × vx × (1 + 1/(1+2d²))

cp5_reward_goal_proximity        → (1-tanh(d/5)) + (1-tanh(d/1))  [dual-scale]

cp5_reward_goal_reached          → 1.0 if dist < 0.3 m else 0.0

cp5_penalty_collision_velocity_scaled → (1+4‖v‖²) × contact_bool

cp5_penalty_stuck                → 1.0 if max_disp < 0.1m AND forward_cmd

cp5_reward_heading               → cos(heading_error)

cp5_penalty_smoothness           → ‖action_t - action_{t-1}‖

CP6 reward functions:

cp6_reward_navigation_core       → exp(-Δfwd²/σ) × exp(-vy²/σ) × exp(-Δθ²/σ)

cp6_reward_path_following        → exp(-dist_to_path² / 1.0)

cp6_penalty_graduated_clearance  → sum over obstacles of exp(-d_i² / 0.25)

cp6_reward_slow_near_goal        → (1 - speed.clamp(1)) × 𝟙[d < 1.5m]

cp6_penalty_stuck_v2             → stuck AND NOT near_goal

cp6_penalty_smoothness_2nd_order → ‖action_t - 2×action_{t-1} + action_{t-2}‖

mdp/events.py

Event functions called on episode reset. Two key ones for CP6/CP6.5:

cp6_randomize_obstacles_and_replan(env, env_ids, position_range)

CP6 only. On reset: shifts ALL 9 obstacles by a shared random (dx, dy),

runs A* to replan path, broadcasts waypoints to all envs. One-time log.

cp65_reset_with_generated_scene(env, env_ids, ramp_iterations=40_000)

CP6.5 only. On reset:

1. Calls env._curriculum.step() to advance difficulty

2. Calls generate_scene() → (obstacles, waypoints, goal_xy)

3. Calls apply_scene_to_env() to move rigid objects in sim

4. Stores waypoints as env._cp5_waypoints (E, W, 2) world tensor

5. Resets wp_idx, prev_dist, action history for resetting envs

6. Rate-limited log (1 per 60s) showing curriculum progress + goal

mdp/terminations.py

Termination functions. The one that matters for navigation:

cp6_goal_reached(env, threshold=0.1) → BoolTensor (E,)

True when robot is within 0.1m of env._cp5_waypoints[:, -1, :].

The last waypoint is ALWAYS the goal (A* ends there, generator ends there).

Works for both fixed goal (CP6) and random goal (CP6.5) without changes.

mdp/__init__.py

Exports everything. Key import chain: wildcard from isaaclab.envs.mdp (base locomotion functions) plus explicit imports of all navigation-specific functions. This lets navigation_env_cfg.py do: import mdp as nav_mdp.

models/ — Neural Network Architecture

models/navigation_policy.py

Defines NavigationPolicy (actor) and NavigationValue (critic) for skrl PPO.

NavigationPolicy  (inherits GaussianMixin, Model)

│

├── CNN branch — processes 40×40 occupancy grid

│     Conv2d(1→16,  5×5, stride=2, pad=2)   40×40 → 20×20  [ELU]

│     Conv2d(16→32, 3×3, stride=2, pad=1)   20×20 → 10×10  [ELU]

│     Conv2d(32→32, 3×3, stride=1, pad=1)   10×10 → 10×10  [ELU]

│     Flatten → 3200

│     Linear(3200→128) [ELU]

│     Linear(128→64)   [ELU]

│     Output: (B, 64)

│

├── MLP branch — processes 15 scalar observations

│     Linear(15→64) [ELU]

│     Linear(64→32) [ELU]

│     Output: (B, 32)

│

├── Fusion head

│     cat(64, 32) → (B, 96)

│     Linear(96→128) [ELU]

│     Linear(128→64) [ELU]

│     Linear(64→3)

│     Output: (B, 3)  ← raw logits

│

└── Output squashing

vx, vy  → tanh(raw[:, :2])        ∈ [-1, 1] m/s

heading → π × tanh(raw[:, 2:3])   ∈ [-π, π] rad

NavigationValue  (same CNN+MLP, Linear(64→1) instead of Linear(64→3))

Key skrl API note:

compute(inputs, role) reads inputs["observations"]  (NOT inputs["states"])

Returns: (mean_actions, {"log_std": self.log_std_parameter})

GaussianMixin uses this dict key — returning 3-tuple causes KeyError.

planning/ — A* Path Planning (CP5 / CP6)

Used in CP5 and CP6. CP6.5 has its own A* embedded in scene_generator.py so the planning/ module is NOT called during CP6.5 training.

planning/global_grid.py

Builds a 2D binary occupancy grid from env.scene rigid object positions. Robot-centred: origin = robot_pos_w - half_grid_size.

build_global_grid(env, grid_resolution=0.2, grid_size=200)

→ (grid, origin, obstacle_info)

Grid convention:

row ↔ Y axis    (row 0 = min Y = origin_y)

col ↔ X axis    (col 0 = min X = origin_x)

Each obstacle inflated by 0.30m (C-space mapping)

Robot half-width 0.155m + safety 0.145m = 0.30m total

BUG FIXED: origin was hardcoded to (0,0) — robot at (35,-35) was

completely outside the grid. Fixed to: origin = robot_pos - half_grid.

planning/astar.py

8-connected A* implementation on a 2D numpy grid. Diagonal cost = sqrt(2) for proper Euclidean distance.

planning/planner.py

Wraps global_grid.py + astar.py into a single AStarPlanner class.

AStarPlanner.plan(start_world_xy, goal_world_xy) → list of (x, y)

Returns waypoints in world coords, downsampled to every 5th cell.

All print spam removed (was flooding terminal during training).

scene/ — Scene Generation and Curriculum (CP6.5)

New in CP6.5. Replaces A* replanning with obstacles-first generation. These files have ZERO Isaac Lab imports — they run standalone.

scene/scene_generator.py

Core generator. Obstacles placed randomly, BFS verifies connectivity, A* finds optimal path — all in local env coordinates.

GO2_WIDTH   = 0.31 m   (actual Go2 body width)

SAFETY_MARGIN = 0.10 m (per-side clearance)

MIN_GAP     = 0.51 m   (0.31 + 2×0.10 — just squeezable)

random_goal(start_xy, dist_range, angle_range) → (gx, gy)

Random goal at variable distance (5-10m) and angle (±46°).

generate_scene(start_xy, goal_xy, num_obstacles, min_gap_width, ...)

→ (obstacles, waypoints, goal_xy)

Algorithm:

1. Build empty occupancy grid (0.1m resolution)

2. For each obstacle attempt:

a. Random position, size in arena

b. Reject if too close to start/goal (< 1.2m)

c. Reject if overlaps existing obstacle

d. Stamp obstacle + C-space inflation onto test_grid

e. BFS: if start↔goal still connected → accept, update grid

3. Run A* on final grid → path_cells

4. Downsample to ~1m waypoint spacing

Returns: obstacles list, waypoints list, goal_xy tuple

apply_scene_to_env(env, obstacles, device) → None

Moves Isaac Lab RigidObjects to generated positions.

Broadcasts local coords to world: world_pos = env_origin + local_pos

Parks unused objects at y=1000m.

torch imported lazily (only needed inside sim).

_stamp(), _bfs_connected(), _astar() — internal helpers

scene/curriculum.py

5-axis linear curriculum driven by call count. Training: step() every reset. Viz: set current_iteration directly.

NavigationCurriculum(ramp_iterations=40_000)

.step()                → increment current_iteration

.get_difficulty()      → dict with 5 keys:

min_gap_width: 2.00m → 0.51m   (corridor tightness)

num_obstacles: 3     → 12      (scene density)

arena_padding: 3.0m  → 1.5m   (how close obstacles are to path)

goal_dist:    (6,7)  → (5,10)  (goal distance spread in m)

goal_angle:   ±0.3   → ±0.8   (goal angle spread in rad)

.progress_str()        → human-readable one-liner for logs

Timing math:

512 envs × 120s episodes / 0.2s nav_dt = 600 steps/episode/env

Events per training iter ≈ 512 × 24 / 600 ≈ 20

ramp_iterations=40_000 ≈ 40_000/20 = 2000 training iterations

navigation_base_env_cfg.py — Base Scene

Defines the base scene: terrain (flat plane), robot (Go2 URDF), 9 obstacle rigid bodies (cube_01..06, cyl_01..03), contact force sensor, RayCaster camera, height scanner. All configs live here as @configclass.

control/ and managers/ — Legacy CP4 Files

control/waypoint_controller.py: PD-based pure-pursuit controller from CP4. Not used in CP5+ training but kept as a CP4 baseline for comparison.

managers/: Legacy manager-based wrappers from early CP development. Not used in CP5+ — replaced by mdp/ functions.

scripts/ — Training and Evaluation

scripts/train.py

Universal training script. Works with any registered task. Key features: auto-detects log directory from task name, bypasses skrl Runner (directly builds PPO + SequentialTrainer + NavigationPolicy + NavigationValue), loads pre-trained checkpoint if available.

python3 scripts/train.py --task NeuroGait-Navigation-CP65-v0 --num_envs 512 --headless

Log directory: logs/skrl/neurogait_navigation_cp65_v0/YYYY-MM-DD_HH-MM-SS/

scripts/test_reward_scales.py

Loads a task config, runs random actions for 200 steps, prints mean/std of every reward term. Use BEFORE training to check reward magnitudes are balanced.

python3 scripts/test_reward_scales.py --task NeuroGait-Navigation-CP65-v0 --num_envs 64 --headless

scripts/cp5/play.py

Runs a trained checkpoint in simulation with rendering. Loads NavigationPolicy from .pt file, runs inference loop.

scripts/cp6/eval_metrics.py

50-episode evaluator: computes success rate, path efficiency (A* path length / actual path length), collision count, steps-to-goal. Outputs JSON summary and optional comparison table with CP5 baseline.

scripts/cp6/visualize_generated_scenes.py

Standalone visualization: 6-panel matplotlib figure showing curriculum progression (0% → 100%). No Isaac Sim needed. Saves to maps/cp6_5_obstacles_first_difficulty.png.

python3 scripts/cp6/visualize_generated_scenes.py

Section 3 — The Evolution: CP5 → CP6 → CP6.5

| What | CP5 | CP6 | CP6.5 |
| --- | --- | --- | --- |
| Task ID | CP5-v0 | CP6-v0 | CP65-v0 ← train this |
| Scene | Fixed 9 obstacles | Same 9, shifted ±1.5m | Random obstacles-first generator |
| Goal | Fixed local (8, 0) | Fixed local (8, 0) | Random: 5-10m, ±46° |
| A* timing | Runs once at init | Replans each reset batch | Embedded in generator |
| Traversability | Not guaranteed | Not guaranteed (fallback) | BFS-verified by construction |
| Rewards | 7 terms, additive | 9 terms, multiplicative core | Same as CP6 (unchanged) |
| Curriculum | None | None | 5-axis ramp over 40k resets |
| Why it fails | Crab-walk, no stop | A* spam, fixed goal | N/A (current) |

Why obstacles-first is better than path-first

Path-first (CP6.5 v1 — discarded):

1. Generate curvy cubic-spline path start→goal

2. Place obstacles OUTSIDE the protected corridor

Problem: obstacles are DECORATION — they never block the path

path curves for no reason (A* would take straighter route)

training scenes don't look like deployment scenes

Obstacles-first (CP6.5 v2 — current):

1. Place obstacles randomly in tight arena around start→goal

2. BFS flood-fill after each: reject if path blocked

3. A* on final grid → SAME algorithm as deployment

Result: obstacles CREATE gaps the policy must thread

A* path is tight and realistic

training = deployment

Section 4 — Data Flow: One Complete env.step()

RL trainer (skrl PPO) calls env.step(action)

action shape: (512, 3) = [vx, vy, heading] per env

Inside env.step():

┌── ActionManager ──────────────────────────────────────────────────────┐

│  PreTrainedPolicyAction receives action (512, 3)                       │

│  Stores as _raw_actions — rewires velocity_commands obs slot           │

│  Builds internal 235-dim locomotion obs (joint pos, vel, body vel ...) │

│  Runs frozen loco policy: 235 → 12 joint targets                      │

│  PD controllers: targets → torques → applied to Go2                   │

│                                                                         │

│  Inner loop × 40 physics steps (50 Hz physics × 0.02s = 1 step):      │

│    PhysX integrates: F=ma → new joint pos, vel, body state            │

└────────────────────────────────────────────────────────────────────────┘

After 40 physics steps = 1 navigation step (0.2s):

┌── ObservationManager ─────────────────────────────────────────────────┐

│  occupancy_grid_obs_cp5():                                              │

│    depth camera → threshold [0.05, 2.0m] → project to 40×40 grid     │

│    robot at (row=10, col=20), 0.2m cells, 8m×8m coverage              │

│    → (512, 1600) float32 tensor                                         │

│                                                                         │

│  future_waypoints_obs():                                                │

│    read env._cp5_waypoints[env_id, wp_idx:wp_idx+3]                   │

│    transform to body frame, compute distances                          │

│    advance wp_idx if robot within 0.3m of current waypoint            │

│    → (512, 9) float32 tensor                                            │

│                                                                         │

│  robot_velocity_obs() → (512, 3)                                        │

│  gravity_obs()        → (512, 3)                                        │

│  concatenate → (512, 1615) obs tensor                                   │

└────────────────────────────────────────────────────────────────────────┘

┌── RewardManager ──────────────────────────────────────────────────────┐

│  navigation_core:  r_fwd × r_lat × r_hdg (multiplicative)             │

│  path_following:   exp(-dist_to_path² / 1.0)                           │

│  goal_proximity:   dual-scale tanh to final waypoint                   │

│  goal_reached:     1.0 if dist < 0.3m                                  │

│  slow_near_goal:   (1-speed) × 𝟙[d < 1.5m]                            │

│  graduated_clearance: -exp per close obstacle                          │

│  collision:        -(1+4‖v‖²) × contact_bool                          │

│  stuck:            -𝟙[no_movement AND fwd_command AND not_near_goal]  │

│  smoothness:       -‖Δaction + Δaction_prev‖  (2nd order jerk)         │

│  total_reward = Σ weight_i × term_i  → (512,) float32                 │

└────────────────────────────────────────────────────────────────────────┘

┌── TerminationManager ─────────────────────────────────────────────────┐

│  cp6_goal_reached:   dist_to_final_waypoint < 0.1m → done             │

│  time_out:           step_count > max_episode_length → done           │

└────────────────────────────────────────────────────────────────────────┘

Return to trainer: (obs, reward, terminated, truncated, info)

PPO updates policy every 24 × 512 = 12,288 nav steps

Section 5 — Data Flow: Episode Reset (CP6.5)

When any env terminates, Isaac Lab calls cp65_reset_with_generated_scene(env, env_ids)

Step 1: Curriculum

env._curriculum.step()         → current_iteration++

diff = curriculum.get_difficulty()

→ {min_gap_width, num_obstacles, arena_padding, goal_dist, goal_angle}

Step 2: Scene generation (pure Python, no GPU, ~1ms)

obstacles, waypoints, goal_xy = generate_scene(

start_xy=(0,0), goal_xy=None, **diff

)

Internally:

a. Random goal via random_goal() — variable distance + angle

b. Build empty grid (0.1m/cell) around tight arena

c. Try placing num_obstacles random obstacles:

- Reject if too close to start/goal

- Reject if overlaps existing obstacle

- BFS check: if start↔goal still reachable → accept

d. A* on final grid → path_cells

e. Downsample to ~1m waypoint spacing → waypoints list

Step 3: Apply to sim

apply_scene_to_env(env, obstacles, device)

For each of 9 scene rigid objects:

if i < len(obstacles): world_pos = env_origin + local_pos

else:                  world_pos[:, 1] = 1000m  (park it far away)

write_root_pose_to_sim(pose, all_env_ids)

Step 4: Store waypoints

local_wp = torch.tensor(waypoints)           # (W, 2) local

env._cp5_waypoints = local_wp + env_origins  # (E, W, 2) world

# Last entry _cp5_waypoints[:, -1, :] = goal world pos

# → cp6_goal_reached termination works automatically

Step 5: Reset tracking state for env_ids

env._cp5_wp_idx[env_ids]      = 0

env._cp5_prev_dist[env_ids]   = inf

env._cp5_prev_action[env_ids] = 0

env._cp5_pos_history[env_ids] = 0

Step 6: Rate-limited log (max 1 per 60s)

[CP6.5] Curriculum 35%: gap=1.51m, obs=6, pad=2.5m, angle=±0.5rad

goal=(3.2, 4.8) | 9 waypoints | 6 obstacles

Section 6 — Reward Functions: Why Each Term

All CP6 reward terms. CP5 is a subset (7 of these, with different weights).

cp6_reward_navigation_core  weight=+10.0

r = exp(-Δfwd²/0.25) × exp(-3vy²) × exp(-Δθ²/0.25)

Forward: Gaussian around target speed 0.8 m/s in the goal direction. Lateral: penalises sideways crab-walk (Go2 should face forward). Heading: penalises facing away from current waypoint. MULTIPLICATIVE: if ANY component is ~0, the whole reward is ~0. This is why CP5 crab-walked: additive rewards let the robot earn goal_proximity while drifting sideways.

Source: Miki et al. 2022 (Science Robotics) — orthogonal velocity concept.

Too high: robot sprints toward waypoint ignoring obstacles. Too low: policy ignores direction.

cp6_reward_path_following  weight=+0.5

r = exp(-min_dist_to_path² / 1.0)

Gaussian proximity to the FULL A*/generator path, not just current waypoint. Teaches robot to thread THROUGH obstacle gaps instead of going around the entire obstacle field.

Weight history: 5.0 → 1.0 → 0.5 (too high caused path-hugging, not goal-seeking).

cp5_reward_goal_proximity  weight=+0.1

r = (1 - tanh(d/5)) + (1 - tanh(d/1))

Dual scale: 5m scale gives global gradient (robot always has incentive to get closer). 1m scale gives fine-grained gradient near goal. Uses FINAL waypoint, not current — persistent gradient that never resets.

Source: Li et al. 2025, Eq. 1.

Small weight (0.1) — just a shaping signal, not the primary reward.

cp5_reward_goal_reached  weight=+50.0

r = 1.0 if dist < 0.3m else 0.0  (CP5: 0.3m, CP6: 0.1m threshold)

Large sparse bonus at episode success. Without this, the policy has no incentive to actually REACH the goal vs circling near it. Weight 50.0 makes it worth ~5 seconds of max navigation_core reward.

Source: X-Nav 2025 — one-time success signal.

cp6_reward_slow_near_goal  weight=+3.0

r = (1 - speed.clamp(1)) × 𝟙[dist < 1.5m]

Near goal: reward is higher when robot is SLOW. Without this: robot oscillates past the goal at full speed, never triggering the 0.1m threshold. With this: robot decelerates to dock.

cp6_penalty_graduated_clearance  weight=-0.05

r = Σᵢ exp(-dᵢ² / 0.25)  summed over all obstacles

Light continuous penalty for being close to obstacles. Teaches wall-hugging avoidance without blocking progress. Weight history: -1.0 → -0.05. At -1.0 it was 1147× dominant because depth camera fires non-NaN values every step even in open space.

cp5_penalty_collision_velocity_scaled  weight=-1.5

r = (1 + 4(vx² + vy² + ωz²)) × 𝟙[base_contact_force > 1N]

Velocity-scaled: crashing at high speed costs more than gentle contact. sensor_cfg MUST specify body_names='base' (trunk only). If '.*' is used, all bodies are selected and the mask logic fails silently.

cp6_penalty_stuck_v2  weight=-0.3

r = 𝟙[max_disp < 0.1m AND fwd_cmd AND NOT near_goal]

Fires when robot commands forward motion but doesn't move. Near-goal exemption: near the goal, slow speed is desired (see slow_near_goal). Without near-goal patch: penalty fires right when the robot should be docking.

Source: SEA-Nav 2026, Table III with near-goal patch.

cp6_penalty_smoothness_2nd_order  weight=-1.0

r = ‖action_t - 2×action_{t-1} + action_{t-2}‖  (jerk = 2nd derivative)

Second-order penalty (vs CP5's first-order). Penalises change in velocity change, not just velocity change. Produces smoother, more physically plausible motions.

Section 7 — CNN+MLP Network: Layer by Layer

Full forward pass with actual tensor shapes (batch size B = 512):

Input obs: (B, 1615)

Split at index 1600:

grid_flat  = obs[:, :1600]    → (B, 1600)

scalars    = obs[:, 1600:]    → (B, 15)

━━━ CNN BRANCH ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

grid_2d = grid_flat.view(B, 1, 40, 40)          → (B, 1, 40, 40)

Conv2d(1→16, k=5, stride=2, pad=2)  [ELU]

B×1×40×40 → B×16×20×20

Conv2d(16→32, k=3, stride=2, pad=1) [ELU]

B×16×20×20 → B×32×10×10

Conv2d(32→32, k=3, stride=1, pad=1) [ELU]

B×32×10×10 → B×32×10×10

Flatten

B×32×10×10 → (B, 3200)

Linear(3200→128) [ELU]             → (B, 128)

Linear(128→64)   [ELU]             → (B, 64)   ← grid_feat

━━━ MLP BRANCH ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

scalars = obs[:, 1600:]

[1600:1609] 3 future waypoints × 3 (dx_body, dy_body, dist)

[1609:1612] robot velocity [vx, vy, yaw_rate] in body frame

[1612:1615] projected gravity vector [gx, gy, gz]

Linear(15→64) [ELU]                → (B, 64)

Linear(64→32) [ELU]                → (B, 32)   ← scalar_feat

━━━ FUSION ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

merged = cat([grid_feat, scalar_feat], dim=-1)  → (B, 96)

Linear(96→128)  [ELU]              → (B, 128)

Linear(128→64)  [ELU]              → (B, 64)

Linear(64→3)                       → (B, 3)    ← raw logits

━━━ OUTPUT SQUASHING ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

vx_vy   = tanh(raw[:, :2])        → [-1.0, 1.0] m/s

heading = π × tanh(raw[:, 2:3])   → [-π, π] rad

→ (B, 3)  = [vx, vy, heading] per env

Total parameters: ~700K  (CNN: ~590K, FC: ~110K)

The CNN sees SPATIAL obstacle layout. The MLP sees WHERE TO GO (waypoints) and HOW FAST I'M MOVING (velocity). Fusing them lets the policy decide: 'given this obstacle field AND these waypoints, what velocity should I command?'

Section 8 — Bugs and Lessons Learned

1. heading_command vs yaw_rate mismatch (CP5 init)

Symptom:

Robot spun in place.

Root cause:

Navigation action[2] was interpreted as yaw_rate but locomotion policy expected a heading angle. Command interface mismatch.

Fix + lesson:

Changed action[2] to mean absolute heading (not yaw rate). Locomotion policy converts heading → yaw error internally.

2. Camera GPU overflow (CP5 init)

Symptom:

CUDA OOM on first reset.

Root cause:

RayCasterCamera output was stored full-resolution; with 512 envs × 200×200 depth image the buffer overflowed.

Fix + lesson:

Switched to 40×40 asymmetric grid (see concept/06_asymmetric_grid.md). Downsampling done on GPU before storing.

3. Reward imbalance — smoothness 33× dominant (CP5 init)

Symptom:

Policy refused to move — any action was immediately penalized.

Root cause:

smoothness weight was -1.0 on a first-order penalty. Random actions have ‖Δa‖ ≈ 1.0 per step, so penalty was ≈ 1.0 while max reward was ≈ 0.03. Policy learned to stay still.

Fix + lesson:

Reduced smoothness to -0.01 (regularizer only). Run test_reward_scales.py BEFORE every training run to catch this class of bug.

4. Progress always zero — prev_dist reset every step (CP5 init)

Symptom:

velocity_toward_goal reward was always ~0.

Root cause:

prev_dist was being reset to inf every step instead of persisting. The progress term (d_prev - d_curr) was always near zero.

Fix + lesson:

Fixed state management: prev_dist only resets on episode reset (in EventTerm), not on every step.

5. render_interval < decimation (CP5 init)

Symptom:

render_interval error from Isaac Lab on startup.

Root cause:

render_interval must be a multiple of decimation (40). Had set render_interval=4 which is less than decimation.

Fix + lesson:

Set render_interval=40 (= one nav step). Documentation was wrong.

6. skrl Runner expects class not instance (CP5 train)

Symptom:

TypeError: Runner expects model class, got NavigationPolicy instance.

Root cause:

PPORunnerCfg requires passing the CLASS (NavigationPolicy) not an instantiated object. Runner instantiates it internally with env spaces.

Fix + lesson:

Bypassed Runner entirely: use PPO + SequentialTrainer directly, pass model instances. Gives full control over model creation.

7. inputs['states'] returns None in skrl (CP5 train)

Symptom:

model.compute() received None for the observation tensor.

Root cause:

skrl passes inputs['observations'] for on-policy PPO, not inputs['states']. Had written self.policy(inputs['states']) following wrong documentation.

Fix + lesson:

Changed to inputs['observations'] in NavigationPolicy.compute().

8. Model on CPU, input on GPU (CP5 play)

Symptom:

RuntimeError: Expected all tensors on same device.

Root cause:

play.py loaded checkpoint on CPU. env.step() returns obs on CUDA. model(obs.cpu()) worked but was slow and easy to miss.

Fix + lesson:

Added model.to(device) immediately after load_state_dict(). Check device placement whenever loading a checkpoint.

9. policy.act() returns 2 values not 3 (CP5 play)

Symptom:

ValueError: too many values to unpack.

Root cause:

skrl GaussianMixin.act() returns (actions, log_prob) in eval mode, not (actions, log_prob, outputs) like training mode.

Fix + lesson:

Unpacked correctly: actions, _ = policy.act(inputs, role='policy').

10. A* grid at wrong location (CP6 BUG 1)

Symptom:

A* reports 'no path found' for every env. Robot gets straight-line fallback.

Root cause:

Grid origin was hardcoded to (0,0). With env_spacing=12m and 512 envs, robots are placed at e.g. (35, -35). The 8m×8m grid around (0,0) doesn't contain the robot at all.

Fix + lesson:

Fixed: origin = robot_pos_w[0, :2] - half_grid_size. The grid is now centred on the robot's actual world position.

11. graduated_clearance 1147× dominant (CP6 BUG 2)

Symptom:

Policy ran away from all obstacles and refused to approach them.

Root cause:

Weight was -1.0. Depth camera fires non-NaN values every step even in open space (ambient depth values ~1.5m). With 9 obstacles each contributing exp(-d²/0.25) ≈ 0.127, total penalty was ~1.14 per step. navigation_core max ≈ 0.001.

Fix + lesson:

Reduced weight to -0.05 (22× reduction). Always test graduated_clearance with test_reward_scales.py before training.

12. Robot crab-walks sideways (CP5 result)

Symptom:

Policy translated laterally toward goal instead of facing it.

Root cause:

CP5 used additive rewards. goal_proximity rewarded getting closer regardless of heading. vy was unconstrained so sideways was fine.

Fix + lesson:

CP6 multiplicative core: r_lateral = exp(-3vy²) collapses the entire navigation_core reward when vy ≠ 0.

13. Robot takes long detours (CP5 result)

Symptom:

Robot went around the entire obstacle field instead of through gaps.

Root cause:

No reward for staying near the A* path. Robot maximized goal_proximity by taking the least-resistance route (around all obstacles).

Fix + lesson:

Added cp6_reward_path_following: rewards proximity to the A* path, which threads THROUGH the gaps.

14. Robot doesn't stop at goal, oscillates (CP5 result)

Symptom:

Robot reaches 0.5m from goal then oscillates back and forth.

Root cause:

No deceleration incentive near goal. goal_reached sparse bonus requires < 0.3m but robot overshoots at full speed.

Fix + lesson:

Added cp6_reward_slow_near_goal (weight +3.0). Also tightened goal_reached threshold from 0.3m to 0.1m.

15. Path-first generator: obstacles are decoration (CP6.5 v1)

Symptom:

Visualization showed A* path was basically a straight line; obstacles were all far from the path.

Root cause:

Generating path first then placing obstacles OUTSIDE the corridor means obstacles can never be near the path. The curriculum's corridor narrowing had no effect on difficulty.

Fix + lesson:

Switched to obstacles-first: place obstacles first, BFS checks connectivity, A* plans through the actual gaps.

16. Fixed goal: policy memorizes direction (CP6.5 v1)

Symptom:

Policy learned to always move in the +X direction regardless of where obstacles or waypoints were.

Root cause:

Goal was always at local (8, 0) — always straight ahead. With enough training, policy learned 'go right' as a shortcut.

Fix + lesson:

Added random_goal(): variable distance (5-10m) and angle (±46°). Policy MUST read the waypoint obs to know where to go.

Section 9 — File Dependency Graph

CONFIGURATION LAYER

───────────────────

navigation_env_cfg.py

├── imports: isaaclab.managers (EventTerm, ObsTerm, RewTerm, DoneTerm)

├── imports: .cp5_rewards.CP5RewardsCfg

├── imports: .cp6_rewards.CP6RewardsCfg

├── imports: mdp as nav_mdp  (references functions by name in *Cfg objects)

└── registered as gym tasks via config/go2/__init__.py

cp5_rewards.py / cp6_rewards.py

└── imports: mdp as nav_mdp  (references reward functions)

config/go2/__init__.py

└── imports: navigation_env_cfg (indirectly via gym string refs)

COMPUTATION LAYER (mdp/)

────────────────────────

mdp/__init__.py

├── wildcard from isaaclab.envs.mdp  (locomotion base functions)

├── from .observations import: occupancy_grid_obs_cp5, future_waypoints_obs,

│                               robot_velocity_obs, _cp5_init_waypoint_state, ...

├── from .rewards import: all cp5_* and cp6_* reward functions

├── from .terminations import: cp6_goal_reached

└── from .events import: cp6_randomize_obstacles_and_replan,

cp65_reset_with_generated_scene

mdp/observations.py

├── imports: planning.global_grid (for A* init)

├── imports: planning.planner (for A* init)

└── _cp5_init_waypoint_state SKIPPED if env._cp5_waypoints already exists

(generator sets it first → A* never called in CP6.5)

mdp/rewards.py

├── imports: observations._cp5_init_waypoint_state (called every step)

└── reads env._cp5_waypoints, env._cp5_wp_idx, env._cp5_prev_dist (env attrs)

mdp/events.py

├── CP6:   imports planning.global_grid, planning.planner

└── CP6.5: imports scene.scene_generator, scene.curriculum

SCENE GENERATION (scene/)

──────────────────────────

scene/scene_generator.py

└── NO PROJECT IMPORTS (standalone — numpy + scipy only)

torch imported lazily (inside apply_scene_to_env only)

scene/curriculum.py

└── imports scene.scene_generator.MIN_GAP  (0.51m constant)

scene/__init__.py

└── re-exports: generate_scene, apply_scene_to_env, NavigationCurriculum, ...

NEURAL NETWORK (models/)

────────────────────────

models/navigation_policy.py

└── imports: torch, torch.nn, skrl.models.torch (Model, GaussianMixin, DeterministicMixin)

NO project imports — can be imported standalone

PLANNING (planning/)

──────────────────────

planning/global_grid.py   → imports: numpy

planning/astar.py         → imports: numpy, heapq

planning/planner.py       → imports: planning.global_grid, planning.astar

SCRIPTS

───────

scripts/train.py

├── imports: models.navigation_policy.NavigationPolicy, NavigationValue

├── uses: gym.make() → navigation_env_cfg chain

└── imports: skrl (PPO, SequentialTrainer)

scripts/test_reward_scales.py

├── uses: gym.make() → navigation_env_cfg chain

└── dynamic task loading via importlib

scripts/cp6/visualize_generated_scenes.py

├── imports: scene.scene_generator (direct path, not through neurogait package)

└── imports: scene.curriculum

NO Isaac Sim dependency — runs with plain python3

ISOLATION NOTES

───────────────

• scene/ can be imported with plain python3 (no Isaac Sim, no pxr)

• models/ can be imported without Isaac Sim (needs torch + skrl)

• Everything in mdp/ needs Isaac Lab (isaaclab.envs, isaaclab.assets, etc.)

• Never import through neurogait.__init__ in standalone scripts

(it cascades into isaaclab_tasks → isaaclab → pxr → GPU drivers)

Section 10 — Quick Reference

Key Environment Attributes (set on env object at runtime)

env._cp5_waypoints     (E, W, 2) float32  world coords of full A*/gen path

env._cp5_wp_idx        (E,)      long     current waypoint index per env

env._cp5_prev_dist     (E,)      float32  distance to wp from previous step

env._cp5_prev_action   (E, 3)    float32  previous nav action [vx,vy,hdg]

env._cp5_pos_history   (E,20,2)  float32  position ring buffer for stuck detection

env._cp5_pos_hist_idx  int       current write index into ring buffer

env._cp6_prev_action_1 (E, 3)    float32  for 2nd-order smoothness (CP6)

env._cp6_prev_action_2 (E, 3)    float32  for 2nd-order smoothness (CP6)

env._curriculum        NavigationCurriculum  scene difficulty tracker (CP6.5)

Key Config Parameters

| Parameter | Value | Where | Why |
| --- | --- | --- | --- |
| decimation | 40 | navigation_env_cfg.py | Nav policy runs at 5Hz (50Hz/40) |
| episode_length_s | 120.0 | CP6EnvCfg | 6 minutes per episode |
| grid_size_m | 8.0 | observations.py | 8m × 8m occupancy coverage |
| n_cells | 40 | observations.py | 0.2m/cell, 1600 total |
| robot_row | 10 | observations.py | 2m behind robot, 6m ahead (asymmetric) |
| wp_advance | 0.3m | observations.py | advance waypoint when within 0.3m |
| goal_threshold | 0.1m | terminations.py | tight goal: 0.1m from last waypoint |
| ramp_iterations | 40_000 | events.py | ≈2000 training iters for full difficulty |

Training Checklist

1. Run test_reward_scales.py first — confirm no term is >10× others

2. Run visualize_generated_scenes.py — confirm obstacles are near path

3. Start training: python3 scripts/train.py --task NeuroGait-Navigation-CP65-v0 --num_envs 512 --headless

4. Watch first 50 iterations: navigation_core should climb, goal_reached near 0 initially

5. The [CP6.5] log line (every 60s) shows curriculum progress

6. At 2000 iterations (~40k resets): full difficulty reached